### ITN Final Skills Exam (PTSA)

#### ITN (Version 7.00) Final PT Skills Assessment (PTSA) Exam Answers

#### Topology

ITN Final PT Skills Assessment (PTSA)

#### Device Names Table

You will receive one of three possible scenarios. In order to use the logical topology diagram that is provided with the instructions, use the device names in the Device Names Table.

| Topology Diagram Name | Your Scenario Name |
| --- | --- |
| R1 | Central-RT |
| S1 | Central-SW |
| PC-A | User-A |
| PC-B | User-B |

#### Addressing Requirements Table

| Item | Requirements |
| --- | --- |
| Network Address | 209.165.201.0/24 |
| LAN 1 subnet host requirements | 29 |
| LAN 2 subnet host requirements | 17 |
| Central-RT G0/0/1 | First host address in LAN 1 subnet |
| Central-RT G0/0/0 | First host address in LAN 2 subnet |
| Central-SW SVI | Second host address in LAN 1 subnet |
| User-A | Last host address in LAN 1 subnet |
| User-B | Last host address in LAN 2 subnet |

### ITN Final PT Skills Assessment (PTSA)

A few things to keep in mind while completing this activity:

- Do not use the browser Back button or close or reload any exam windows during the exam.

- Do not close Packet Tracer when you are done. It will close automatically.

- Click the Submit Assessment button in the browser window to submit your work.

### Assessment Objectives

- Part 1: Build the Network

- Part 2: Develop an IP Addressing Scheme

- Part 3: Configure Basic Device Settings

- Part 4: Configure Security Settings on Central-RT and Central-SW

- Part 5: Configure the Hosts and Verify Connectivity

### Instructions

In this assessment you will configure the Central-RT router and Central-SW switch, as you have done in the activities in this course. You will also connect two PCs using a switch and a router that are in the main wiring closet. You will subnet the 209.165.201.0/24 network to provide IPv4 addresses for two subnets that will support the required number of hosts. The larger subnet (LAN 1) requires 29 hosts and the smaller subnet (LAN 2) requires 17 hosts.

#### Part 1: Build the Network

a. Build the network according to the logical topology by placing the required equipment in the wiring closet equipment rack.
b. Cable the network devices in the closet as shown in the topology diagram.
c. Connect the hosts as shown in the topology diagram.

#### Part 2: Develop an IP Addressing Scheme

In this part of the assessment, you will develop an IP addressing scheme. You will subnet an IPv4 network to create two subnets with the required number of hosts. You will also subnet an IPv6 network. You will then assign the addresses according to the requirements below.

Work with the following information:

- IPv4 Network: 209.165.201.0/24

- Required number of hosts in IPv4 LAN 1: 29

- Required number of hosts in IPv4 LAN 2: 17

a. Record your subnet assignments according to the following requirements.

1) Assign the first IPv4 address of each subnet to a router interface

- LAN 1 is hosted on Central-RT G0/0/1

- LAN 2 is hosted on Central-RT G0/0/0

2) Assign the last IPv4 address of each subnet to the PC NIC.
3) Assign the second IPv4 address of LAN 1 to Central-SW SVI.

#### Part 3: Configure Basic Device Settings

Network devices must be configured over a direct console connection. Connectivity between the hosts should be established.

##### Step 1: Configure Basic Settings

a. Disable DNS lookup on Central-RT and Central-SW
b. Configure router hostname using the name Central-RT.
c. Configure switch hostname using the name Central-SW.
d. Configure an appropriate banner on Central-RT and Central-SW.
e. Allow console logins with the password C@nsPassw!

##### Step 2: Configure Interfaces

a. Configure Central-RT G0/0/0 and G0/0/1 interfaces using the addressing from the previous part of this assessment:

- Interface description

- IPv4 address / subnet mask

b. Configure the Central-SW VLAN 1 SVI interface using the addressing from the previous part of this assessment:

- Interface description

- IPv4 address / subnet mask

- The switch should be reachable from devices on other networks.

#### Part 4: Configure Security Settings on Central-RT and Central-SW

##### Step 1: Configure enhanced password security

a. Configure ThisisaSecret as the encrypted privileged EXEC password
b. Encrypt all plaintext passwords
c. Set minimum password length to 10 on Central-RT.

##### Step 2: Configure SSH on Central-RT and Central-SW

a. Configure netsec.com as the domain name
b. Configure a local user netadmin with the secret password Ci$co12345
c. Set login on vty lines to use local database.
d. Configure the vty lines to accept SSH access only.
e. Generate an RSA crypto key using 1024 bits modulus.

##### Step 3: Secure switch ports on Central-SW

a. Shut down all unused ports on Central-SW.
b. Enter descriptions for all unused switch ports to indicate that they are intentionally shutdown.

#### Part 5: Configure the Hosts and Verify Connectivity

Configure both hosts with the IPv4 addresses that were assigned in Part 2 of this assessment.
ID: 210

### Answers Key - 100% Score

#### Part 1: Build the Network

Placing Switch Central-SW and Router Central-RT to wiring closet equipment rack.

Using Copper Straight-Through cable to connect User-A (FastEthernet0 port) and Central-SW (FastEthernet0/6 port)

Same as above, using Copper Straight-Through cable to connect all devices as shown in the topology diagram.

ITN Final PT Skills Assessment (PTSA)

Turn-on PCs and Router Central-RT

#### Part 2: Develop an IP Addressing Scheme

| Item | Requirements | IPv4 Address |
| --- | --- | --- |
| Network Address | 209.165.201.0/24 |  |
| LAN 1 subnet host requirements | 29 | 209.165.201.0/27 SM: 255.255.255.224 |
| LAN 2 subnet host requirements | 17 | 209.165.201.32/27 SM: 255.255.255.224 |
| Central-RT G0/0/1 | First host address in LAN 1 subnet | 209.165.201.1 |
| Central-RT G0/0/0 | First host address in LAN 2 subnet | 209.165.201.33 |
| Central-SW SVI | Second host address in LAN 1 subnet | 209.165.201.2 |
| User-A | Last host address in LAN 1 subnet | 209.165.201.30 |
| User-B | Last host address in LAN 2 subnet | 209.165.201.62 |

Using line console to connect User-A and Router

Click to User-A --> Terminal app --> click OK

Router Central-RT configuration script

enable

configure terminal

no ip domain-lookup

hostname Central-RT

banner motd #Unauthorized access to this device is prohibited!#

interface g0/0/0

description Connect to Subnet B

ip address 209.165.201.33 255.255.255.224

no shutdown

exit

interface g0/0/1

description Connect to Subnet A

ip address 209.165.201.1 255.255.255.224

no shutdown

exit

enable secret ThisisaSecret

service password-encryption

security passwords min-length 10

ip domain-name netsec.com

username netadmin secret Ci$co12345

line console 0

password C@nsPassw!

login

exit

line vty 0 15

transport input ssh

login local

exit

crypto key generate rsa

1024

exit

copy running-config startup-config

#### Configuration for Switch Central-SW

Then, using Console cable to connect User-B and Switch
To show Console port on Switch, Right click Switch --> Inspect Rear --> Console port

Switch Central-SW configuration script

enable

configure terminal

no ip domain-lookup

hostname Central-SW

banner motd #Unauthorized access to this device is prohibited!#

interface vlan 1

description Switch Subnet A

ip address 209.165.201.2 255.255.255.224

no shutdown

exit

ip default-gateway 209.165.201.1

enable secret ThisisaSecret

service password-encryption

ip domain-name netsec.com

username netadmin secret Ci$co12345

line console 0

password C@nsPassw!

login

exit

line vty 0 15

transport input ssh

login local

exit

crypto key generate rsa

1024

int range f0/1 - 4, f0/7 - 24, g0/1 - 2

description Unused switch ports

shutdown

end

copy running-config startup-config

#### Part 5: Configure the Hosts and Verify Connectivity

On PCs, go to Desktop tab --> IP Configuration menu

| PC-A Network Configuration |  |
| --- | --- |
| IPv4 Address | 209.165.201.30 |
| Subnet Mask | 255.255.255.224 |
| IPv4 Default Gateway | 209.165.201.1 |

| PC-B Network Configuration |  |
| --- | --- |
| IPv4 Address | 209.165.201.62 |
| Subnet Mask | 255.255.255.224 |
| IPv4 Default Gateway | 209.165.201.33 |