NST21022 - Practical for Network Switching and Routing

Department of Information and Communication Technology

Faculty of Technology

Labsheet :07

Reg. Number: SEU/IS/20/ICT/084

Academic Year :2020/2021

Practical No :07

Title: IPv4 VLSM (Variable Length Subnet Mask)

Aim:

• VLSM IP addressing scheme

• Configure devices with IPv4 after subnetting

Task:

• Design an IP scheme

• Subnet the IPv4 address

• Assign IP address to Network devices and verify connectivity

Use “NST21022 Labsheet 07.pka” file Activities

Exercise 01: Subnet the 192.168.1.0/24 network into the appropriate subnets.

- Based on the topology, how many subnets were needed?

- 4

- How many hosts need for each subnet?

- Subnet 01 –82

- Subnet 02 -32

- Subnet 03 -52

- Subnet 04 -2

- How many hosts provide by each subnet?

- Subnet 01 –128

- Subnet 02 -32

- Subnet 03 -64

- Subnet 04 -4

Exercise 02: Fill the subnet table

| Subnet Numbe | Network Address | First Usable Host Address. | Last Usable Host Address. | Broadcast Address. |
| --- | --- | --- | --- | --- |
| 1 | 192.168.1.0 | 192.168.1.1 | 192.168.1.126 | 192.168.1.127 |
| 2 | 192.168.1.128 | 192.168.1.127 | 192.168.1.190 | 192.168.1.191 |
| 3 | 192.168.1.192 | 192.168.1.193 | 192.168.1.222 | 192.168.1.223 |
| 4 | 192.168.1.224 | 192.168.1.225 | 192.168.1.253 | 192.168.1.254 |

Exercise 03: Configure IP address according to following criteria.

1. Fill the addressing table using following guidelines:

a. Assign the first usable IP addresses in each subnet to R1 for the two LAN link and WAN link

b. Assign the first usable IP addresses of the subnet to R2 for the LAN links, assign the last usable IP address for the WAN link

c. Assign the second usable IP address in the attached subnets to the switches

d. Assign the third usable IP address to the PCs in each subnet

Addressing Table

| Devices | Interfaces | IP Addresses | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | G0/0/0 | 192.168.1.1 | 255.255.255.128 |  |
|  | G0/0/1 | 192.168.1.193 | 255.255.255.224 |  |
|  | S0/1/0 | 192.168.1.225 | 255.255.255.240 |  |
| R2 | G0/0/0 | 192.168.1.129 | 255.255.255.192 |  |
|  | S0/1/0 | 192.168.1.138 | 255.255.255.240 |  |
| S1 | VLAN1 | 192.168.1.2 | 255.255.255.128 |  |
| S2 | VLAN1 | 192.168.1.194 | 255.255.255.224 |  |
| S2 | VLAN1 | 192.168.1.130 | 255.255.255.192 |  |
| PC-1 | NIC | 192.168.1.3 | 255.255.255.128 | 192.168.1.1 |
| PC-2 | NIC | 192.168.1.195 | 255.255.255.224 | 192.168.1.193 |
| PC-3 | NIC | 192.168.1.131 | 255.255.255.192 | 192.168.1.129 |

2. Assign IP addresses to network devices and verify connectivity

Discussion

- In this lab session, we explored the concept of Variable Length Subnet Mask (VLSM) and its application in designing efficient IP addressing schemes. The goal was to create a flexible IP addressing plan that maximizes the use of available IP addresses. We started by designing an IP scheme based on the specific requirements for different subnets, using VLSM to allocate varying subnet sizes based on the number of hosts in each network segment. After subnetting the IPv4 address using VLSM, we assigned the appropriate IP addresses to network devices, ensuring each subnet had the correct size. Finally, we verified connectivity between devices, confirming that communication within and between subnets was successful. This session provided hands-on experience in optimizing IP address allocation using VLSM.