NST21022 - Practical for Network Switching and Routing

Department of Information and Communication Technology

Faculty of Technology

Lab sheet :12

Reg. Number: SEU/IS/20/ICT/084

Academic Year :2020/2021

Practical No : 12

Title: Configure VLANs

Aim:

• Configure static and dynamic trucking protocol

• Configure inter-VLAN routing

Task:

• Configure static trucking protocol

• Configure dynamic trucking protocol

• Configure Router-on-a-Stick inter-VLAN routing

Activities

Exercise 01: Configure Static and Dynamic Trucking Protocol Use “NST21022 Lab sheet 12E1.pka” file

Addressing Table

| Device | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| PC-1 | NIC | 192.168.1.10 | 255.255.255.0 |
| PC-2 | NIC | 192.168.2.10 | 255.255.255.0 |
| PC-3 | NIC | 192.168.3.10 | 255.255.255.0 |
| PC-A | NIC | 192.168.1.20 | 255.255.255.0 |
| PC-B | NIC | 192.168.2.20 | 255.255.255.0 |
| PC-C | NIC | 192.168.3.20 | 255.255.255.0 |

- Create additional VLANs on S1, S2 and S3 according to the table below.

| VLAN Number | VLAN Name |
| --- | --- |
| 10 | Red |
| 20 | Green |
| 30 | Blue |
| 999 | Native |

Switch>enable

Switch#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Switch(config)#hostname S1

S1(config)#ip domain-name ccna.coma

S1(config)#vlan 10

S1(config-vlan)#name Red

S1(config-vlan)#vlan 20

S1(config-vlan)#name Green

S1(config-vlan)#vlan 30

S1(config-vlan)#name Blue

S1(config-vlan)#vlan 999

S1(config-vlan)#name Native

S1(config-vlan)#

Switch>enable

Switch#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Switch(config)#hostname S2

S2(config)#ip domain-name ccna.com

S2(config)#vlan 10

S2(config-vlan)#name Res

S2(config-vlan)#vlan 10

S2(config-vlan)#name Red

S2(config-vlan)#vlan 20

S2(config-vlan)#name Green

S2(config-vlan)#vlan 30

S2(config-vlan)#name Blue

S2(config-vlan)#vlan 999

S2(config-vlan)#name Native

S2(config-vlan)#

S3#

S3#enable

S3#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

S3(config)#hostname S3

S3(config)#ip domain-name ccna.com

S3(config)#vlan 10

S3(config-vlan)#name Res

S3(config-vlan)#vlan 10

S3(config-vlan)#name Red

S3(config-vlan)#vlan 20

S3(config-vlan)#name Green

S3(config-vlan)#vlan 30

S3(config-vlan)#name Blue

S3(config-vlan)#vlan 999

S3(config-vlan)#name Native

S3(config-vlan)#

2. Assign Ports to VLANs

| Ports | Assignments | Network |
| --- | --- | --- |
| S2 F0/1 – 8 S3 F0/1 – 8 | VLAN 10 (Red) | 192.168.1.0/24 |
| S2 F0/9 – 16 S3 F0/9 – 16 | VLAN 20 (Green) | 192.168.2.0/24 |
| S2 F0/17 – 24 S3 F0/17 – 24 | VLAN 30 (Blue) | 192.168.3.0/24 |

S2(config)#interface fa0/1

S2(config-if)#int range fa0/1-8

S2(config-if)#int range fa0/1-8

S2(config-if-range)#switchport mode access

S2(config-if-range)#switchport access vlan 10

S2(config-if-range)#exit

S2(config)#interface fa0/11

S2(config-if)#int range fa0/9-16

S2(config-if-range)#switchport mode access

S2(config-if-range)#switchport mode access

S2(config-if-range)#switchport access vlan 20

S2(config-if-range)#exit

S2(config)#interface fa0/21

S2(config-if)#int range fa0/17-24

S2(config-if)#int range fa0/17-24

S2(config-if-range)#switchport mode access

S2(config-if-range)#switchport access vlan 30

S2(config-if-range)#exit

S2(config)#

S3(config)#interface fa0/1

S3(config-if)#int range fa0/1-8

S3(config-if)#int range fa0/1-8

S3(config-if-range)#switchport mode access

S3(config-if-range)#switchport access vlan 10

S3(config-if-range)#exit

S3(config)#interface fa0/11

S3(config-if)#int range fa0/9-16

S3(config-if-range)#switchport mode access

S3(config-if-range)#switchport mode access

S3(config-if-range)#switchport access vlan 20

S3(config-if-range)#exit

S3(config)#interface fa0/21

S3(config-if)#int range fa0/17-24

S3(config-if)#int range fa0/17-24

S3(config-if-range)#switchport mode access

S3(config-if-range)#switchport access vlan 30

S3(config-if-range)#exit

S3(config)#

3. Dynamic trunking protocol (DTP) manages the trunk links between Cisco switches. Currently, all the switchports are in the default trunking mode, which is dynamic auto

a. On S1, configure the trunk link to dynamic desirable on the interface GigabitEthernet 0/1

S1(config)# interface g0/1

S1(config)# switchport mode dynamic desirable

S1(config)#interface g0/1

S1(config-if)#switchport mode dynamic desirable

S1(config-if)#

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/1, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/1, changed state to up

b. On S2, verify that the trunk has been negotiated by issue the show interface trunk command

S2#show interface trunk

Port Mode Encapsulation Status Native vlan

Gig0/1 auto n-802.1q trunking 1

Port Vlans allowed on trunk

Gig0/1 1-1005

Port Vlans allowed and active in management domain

Gig0/1 1,10,20,30,999

Port Vlans in spanning tree forwarding state and not pruned

Gig0/1 1,10,20,30,999

c. For the trunk link between S1 and S3, configure interface GigabitEthernet 0/2 as a static trunk link on S1. In addition, disable DTP negotiation on interface GigabitEthernet 0/2 on S1

S1(config)# interface g0/2

S1(config)# switchport mode trunk

S1(config)# switchport nonegotiate

configure interface GigabitEthernet 0/2 as a static trunk link on S3 also.

S1(config)#

S1(config)#

S1(config)#interface g0/2

S1(config-if)#switchport mode trunk

S1(config-if-range)#switchport trunk native vlan 999

S1(config-if)#switchport nonegotiate

S1(config-if)#exit

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/2, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/2, changed state to up

S1(config)#

S3(config)#interface g0/2

S3(config-if)#switchport mode trunk

S3(config-if)#switchport trunk native vlan 999

S3(config-if)#switchport nonegotiate

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/2, changed state to down

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/2, changed state to up

S3(config-if)#

d. Verify DTP status

S1# show dtp

S1#

S1#show dtp

Global DTP information

Sending DTP Hello packets every 30 seconds

Dynamic Trunk timeout is 300 seconds

1 interfaces using DTP

S1#

S1#

e. Verify trunking is enabled on all the switches

S1# show interface trunk

S1#show interface trunk

Port Mode Encapsulation Status Native vlan

Gig0/1 on 802.1q trunking 999

Gig0/2 on 802.1q trunking 999

Port Vlans allowed on trunk

Gig0/1 1-1005

Gig0/2 1-1005

Port Vlans allowed and active in management domain

Gig0/1 1,10,20,30,999

Gig0/2 1,10,20,30,999

Port Vlans in spanning tree forwarding state and not pruned

Gig0/1 1,10,20,30,999

Gig0/2 1,10,20,30,999

S1#

f. Configure VLAN 999 as the native VLAN for the trunk links on all switches.

S1(config)# interface g0/1

S1(config)# switchport trunk native vlan 999

S1(config)#interface range g0/1-2

S1(config-if-range)#switchport mode trunk

S1(config-if-range)#switchport trunk native vlan 999

S2(config-if)#interface g0/1

S2(config-if)#switchport mode trunk

S2(config-if)#switchport trunk native vlan 999

g. Notice that each PC can ping the other PC that shares the same subnet.

i. PC-1 can ping PC-A

ii. PC-2 can ping PC-B

iii. PC-3 can ping PC-C

Exercise 02: Configure Router-on-a-Stick inter-VLAN routing Use “NST21022 Labsheet 12E2.pka” file

Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | G0/0/0.10 | 192.168.1.1 | 255.255.255.0 | N/A |
|  | G0/0/0.20 | 192.168.2.1 | 255.255.255.0 |  |
|  | G0/0/0.30 | 192.168.3.1 | 255.255.255.0 |  |
| PC-A | NIC | 192.168.1.10 | 255.255.255.0 | 192.168.1.1 |
| PC-1 | NIC | 192.168.1.11 | 255.255.255.0 | 192.168.1.1 |
| PC-B | NIC | 192.168.2.10 | 255.255.255.0 | 192.168.2.1 |
| PC-2 | NIC | 192.168.2.11 | 255.255.255.0 | 192.168.2.1 |
| PC-C | NIC | 192.168.3.10 | 255.255.255.0 | 192.168.3.1 |
| PC-3 | NIC | 192.168.3.11 | 255.255.255.0 | 192.168.3.1 |

- Create and name VLANs on S1

| VLAN Number | VLAN Name |
| --- | --- |
| 10 | Staffs |
| 20 | Students |
| 30 | Guests |

S1>enable

S1#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

S1(config)#vlan 10

S1(config-vlan)#name Staffs

S1(config-vlan)#vlan 20

S1(config-vlan)#name Students

S1(config-vlan)#vlan 30

S1(config-vlan)#name Guests

S1(config-vlan)#

- Assign Ports to VLANs

| Ports | Assignments | Network |
| --- | --- | --- |
| S1 F0/1 – 2 | VLAN 10 (Staffs) | 192.168.1.0/24 |
| S1 F0/11-12 | VLAN 20 (Students) | 192.168.2.0/24 |
| S1 F0/21-22 | VLAN 30 (Guests) | 192.168.3.0/24 |

S1(config)#interface range fa0/1-2

S1(config-if-range)#switchport mode access

S1(config-if-range)#switchport access vlan 10

S1(config-if-range)#interface range fa0/11-12

S1(config-if-range)#switchport mode access

S1(config-if-range)#switchport access vlan 20

S1(config-if-range)#interface range fa0/21-22

S1(config-if-range)#switchport mode access

S1(config-if-range)#switchport access vlan 30

S1(config-if-range)#

- Verify VLAN configuration

S1#show ip interface brief

Interface IP-Address OK? Method Status Protocol

FastEthernet0/1 unassigned YES manual up up

FastEthernet0/2 unassigned YES manual up up

FastEthernet0/3 unassigned YES manual down down

FastEthernet0/4 unassigned YES manual down down

FastEthernet0/5 unassigned YES manual down down

FastEthernet0/6 unassigned YES manual down down

FastEthernet0/7 unassigned YES manual down down

FastEthernet0/8 unassigned YES manual down down

FastEthernet0/9 unassigned YES manual down down

FastEthernet0/10 unassigned YES manual down down

FastEthernet0/11 unassigned YES manual up up

FastEthernet0/12 unassigned YES manual up up

FastEthernet0/13 unassigned YES manual down down

FastEthernet0/14 unassigned YES manual down down

FastEthernet0/15 unassigned YES manual down down

FastEthernet0/16 unassigned YES manual down down

FastEthernet0/17 unassigned YES manual down down

FastEthernet0/18 unassigned YES manual down down

FastEthernet0/19 unassigned YES manual down down

FastEthernet0/20 unassigned YES manual down down

FastEthernet0/21 unassigned YES manual up up

FastEthernet0/22 unassigned YES manual up up

FastEthernet0/23 unassigned YES manual down down

FastEthernet0/24 unassigned YES manual down down

GigabitEthernet0/1 unassigned YES manual down down

GigabitEthernet0/2 unassigned YES manual down down

Vlan1 unassigned YES manual administratively down down

S1#

- 4. Configure sub interfaces on R1 using 802.1Q encapsulation a. Create the sub interfaces G0/0/0.10

- Set the encapsulation type 802.1Q, assign VLAN 10 and assign the correct IP address to the sub interface.

R1(config)# int g0/0/0.10

R1(config-subif)# encapsulation dot1Q 10

R1(config-subif)# ip address 192.168.1.1 255.255.255.0

R1>enable

R1#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

R1(config)#interface g0/0/0.10

R1(config-subif)#encapsulation dot1Q 10

R1(config-subif)#ip address 192.168.1.1 255.255.255.0

R1(config-subif)#no shutdown

R1(config-subif)#

b. Repeat the steps for other 2 sub interfaces.

R1(config-subif)#interface g0/0/0.20

R1(config-subif)#encapsulation dot1Q 20

R1(config-subif)#ip address 192.168.2.1 255.255.255.0

R1(config-subif)#no shutdown

R1(config-subif)#

R1(config-subif)#interface g0/0/0.30

R1(config-subif)#encapsulation dot1Q 30

R1(config-subif)#ip address 192.168.3.1 255.255.255.0

R1(config-subif)#no shutdown

R1(config-subif)#

5. Verify configuration

a. Use show ip interface brief command to verify sub interface configuration.

R1#show ip interface brief

Interface IP-Address OK? Method Status Protocol

GigabitEthernet0/0/0 unassigned YES unset administratively down down

GigabitEthernet0/0/0.10192.168.1.1 YES manual administratively down down

GigabitEthernet0/0/0.20192.168.2.1 YES manual administratively down down

GigabitEthernet0/0/0.30192.168.3.1 YES manual administratively down down

GigabitEthernet0/0/1 unassigned YES unset administratively down down

GigabitEthernet0/0/2 unassigned YES unset administratively down down

Vlan1 unassigned YES unset administratively down down

R1#

b. Enable G0/0/0 interface.

R1#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

R1(config)#

R1(config)#interface g0/0/0

R1(config-if)#no shutdown

R1(config-if)#

6. Try to ping between PCs on different networks.

a. The pings should still fail.

b. Because the router was configured with multiple sub interfaces assigned to different VLANs, the switch port connecting to the router must be configured as a trunk. Enable trucking on interface G0/1.

S1(config)#interface g0/1

S1(config-if)#switchport mode trunk

c. Verify the trunk configuration on S1

S1>en

S1#config

Configuring from terminal, memory, or network [terminal]?

Enter configuration commands, one per line. End with CNTL/Z.

S1(config)#interface g0/1

S1(config-if)#switchport mode trunk

7. Test connectivity.

Discussion

- In this lab session, we focused on configuring both static and dynamic trucking protocols, as well as setting up inter-VLAN routing using the Router-on-a-Stick method. First, we established a static trunk by manually setting the trunk mode on the switch ports to allow VLAN traffic across different switches. Then, for dynamic trunking, we enabled Dynamic Trunking Protocol (DTP), which automatically negotiates the trunking state between switches. Finally, we implemented inter-VLAN routing through Router-on-a-Stick, where a single router interface was used to route traffic between multiple VLANs by creating subinterfaces for each VLAN. This configuration enables efficient communication between VLANs and facilitates network segmentation.