NST21022 - Practical for Network Switching and Routing

Department of Information and Communication Technology

Faculty of Technology

Lab sheet :13

Reg. Number: SEU/IS/20/ICT/084

Academic Year :2020/2021

Practical No :13

Title: Configure VLANs

Aim:

• Getting familiar with Layer-3 Switching and Inter-VLAN Routing

Task:

• Design a network topology

• Adding VLANs to a Switch

• Configure IP addresses

• Configure Layer 3 switch

• Configure Inter-VLAN Routing

Activities

Exercise 01: Design a Network Topology Use “NST21022 Lab sheet 14.pka” file VLAN and

Addressing Table

| Device | VLAN | Name | Network | Device |
| --- | --- | --- | --- | --- |
| S1 | VLAN 10 | Red | 192.168.1.0/24 | VLAN 10 |
|  | VLAN 20 | Green | 192.168.2.0/24 | VLAN 20 |
|  | VLAN 30 | Blue | 192.168.3.0/24 | VLAN 30 |
| S2 | VLAN 40 | Yellow | 192.168.4.0/24 | VLAN 40 |
|  | VLAN 50 | White | 192.168.5.0/24 | VLAN 50 |
|  | VLAN 60 | Black | 192.168.6.0/24 | VLAN 60 |

| Device | Interface | IP Address | VLAN | Device |
| --- | --- | --- | --- | --- |
| Colors. | VLAN 10 | 192.168.1.1/24 | VLAN 10 |  |
|  | VLAN 20 | 192.168.2.1/24 | VLAN 20 |  |
|  | VLAN 30 | 192.168.3.1/24 | VLAN 30 |  |
|  | VLAN 40 | 192.168.4.1/24 | VLAN 40 |  |
|  | VLAN 50 | 192.168.5.1/24 | VLAN 50 |  |
|  | VLAN 60 | 192.168.6.1/24 | VLAN 60 |  |
| PC-1 | NIC | 192.168.1.10/24 | VLAN 10 | PC-1 |
| PC-2 | NIC | 192.168.2.10/24 | VLAN 20 | PC-2 |
| PC-3 | NIC | 192.168.3.10/24 | VLAN 30 | PC-3 |
| PC-4 | NIC | 192.168.4.10/24 | VLAN 40 | PC-4 |
| PC-5 | NIC | 192.168.5.10/24 | VLAN 50 | PC-5 |
| PC-6 | NIC | 192.168.6.10/24 | VLAN 60 | PC-6 |

- Create a network topology with layer 3 switch including another 2, layer 2 switch.

- Create additional VLANs on Colors, S1 and S2 according to the table above.

Colors>enable

Colors#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

Colors(config)#hostname Colors

Colors(config)#

Colors(config)#vlan 10

Colors(config-vlan)#name Red

Colors(config-vlan)#vlan 20

Colors(config-vlan)#name Green

Colors(config-vlan)#vlan 30

Colors(config-vlan)#name Blue

Colors(config-vlan)#vlan 40

Colors(config-vlan)#name Yellow

Colors(config-vlan)#vlan 50

Colors(config-vlan)#name White

Colors(config-vlan)#vlan 60

Colors(config-vlan)#

S1>enable

S1#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

S1(config)#hostname S1

S1(config)#

S1(config)#vlan 10

S1(config-vlan)#name Red

S1(config-vlan)#vlan 20

S1(config-vlan)#name Green

S1(config-vlan)#vlan 30

S1(config-vlan)#name Blue

S1(config-vlan)#

2>enable

S2#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

S2(config)#hostname S2

S2(config)#

S2(config)#vlan 40

S2(config-vlan)#name Yellow

S2(config-vlan)#vlan 50

S2(config-vlan)#name White

S2(config-vlan)#vlan 60

S2(config-vlan)#name Black

S2(config-vlan)#

- Assign Ports to VLANs

S1(config-vlan)#

S1(config-vlan)#exit

S1(config)#interface range f0/1-10

S1(config-if-range)#switchport mode access

S1(config-if-range)#switchport access vlan 10

S1(config-if-range)#

S1(config-if-range)#interface range f0/11-19

S1(config-if-range)#switchport mode access

S1(config-if-range)#switchport access vlan 20

S1(config-if-range)#

S1(config-if-range)#interface range f0/20-24

S1(config-if-range)#switchport mode access

S1(config-if-range)#switchport access vlan 30

S1(config-if-range)#

S2(config)#interface range f0/1-10

S2(config-if-range)#switchport mode access

S2(config-if-range)#switchport access vlan 40

S2(config-if-range)#

S2(config-if-range)#interface range f0/11-20

S2(config-if-range)#switchport mode access

S2(config-if-range)#switchport access vlan 50

S2(config-if-range)#

S2(config-if-range)#interface range f0/21-24

S2(config-if-range)#switchport mode access

S2(config-if-range)#switchport access vlan 60

S2(config-if-range)#

S2(config-if-range)#interface g0/1

S2(config-if)#switchport mode access

S2(config-if)#switchport mode trunk

S2(config-if)#

- Configure Switch Virtual Interface (SVI) on Colors

Colors(config)# interface vlan 10

Colors(config-if)# ip address 192.168.1.1 255.255.255.0

Colors(config)#interface vlan 10

Colors(config-if)#ip address 192.168.1.1 255.255.255.0

Colors(config-if)#no shutdown

Colors(config-if)#

Colors(config-if)#interface vlan 20

Colors(config-if)#ip address 192.168.2.1 255.255.255.0

Colors(config-if)#no shutdown

Colors(config-if)#

Colors(config-if)#interface vlan 30

Colors(config-if)#ip address 192.168.3.1 255.255.255.0

Colors(config-if)#no shutdown

Colors(config-if)#

Colors(config-if)#interface vlan 40

Colors(config-if)#ip address 192.168.4.1 255.255.255.0

Colors(config-if)#no shutdown

Colors(config-if)#

Colors(config-if)#interface vlan 50

Colors(config-if)#ip address 192.168.5.1 255.255.255.0

Colors(config-if)#no shutdown

Colors(config-if)#

Colors(config-if)#interface vlan 60

Colors(config-if)#ip address 192.168.6.1 255.255.255.0

Colors(config-if)#no shutdown

Colors(config-if)#

- Configure trunking on Colors

- Configure interface g0/1 and g0/2

Colors(config)#

Colors(config)#interface range g0/1-2

Colors(config-if-range)#

- Make the interface a static trunk mode

Colors(config-if-range)# switchport mode trunk

Colors(config-if-range)#switchport mode trunk

Colors(config-if-range)#

- Encaplsulate the link with the dot1q protocol

Colors(config-if-range)# switchport trunk encapsulation dot1q

Colors(config)#interface range g0/1-2

Colors(config-if-range)#switchport mode trunk

Colors(config-if-range)#switchport trunk encapsulation dot1q

Colors(config-if-range)#

- Configure trunking in S1 and S2

S1(config)#

S1(config)#interface g0/1

S1(config-if)#switchport mode access

S1(config-if)#switchport mode trunk

S1(config-if)#

S2(config)#interface g0/1

S2(config-if)#switchport mode access

S2(config-if)#switchport mode trunk

S2(config-if)#

- Enable routing in global configuration mode.

Colors(config)#ip routing

Colors(config)#ip routing

Colors(config)#

- Check routing table

Colors#show ip route

Codes: C - connected, S - static, I - IGRP, R - RIP, M - mobile, B - BGP

D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area

N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2

E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP

i - IS-IS, L1 - IS-IS level-1, L2 - IS-IS level-2, ia - IS-IS inter area

* - candidate default, U - per-user static route, o - ODR

P - periodic downloaded static route

Gateway of last resort is not set

C 192.168.1.0/24 is directly connected, Vlan10

C 192.168.2.0/24 is directly connected, Vlan20

C 192.168.3.0/24 is directly connected, Vlan30

C 192.168.4.0/24 is directly connected, Vlan40

C 192.168.5.0/24 is directly connected, Vlan50

C 192.168.6.0/24 is directly connected, Vlan60

Colors#

- Verify connectivity

Discussion

- In this lab session, we focused on understanding Layer-3 switching and configuring inter-VLAN routing. We began by designing a network topology that included multiple VLANs to segment the network and improve security. After setting up the network, we added VLANs to the switch and assigned specific VLAN IDs to relevant switch ports. Next, we configured IP addresses for the devices within each VLAN to enable proper communication. By enabling Layer 3 switching, we were able to route traffic between VLANs directly on the switch without the need for a router. This was done by configuring the switch as a Layer 3 device and setting up inter-VLAN routing, ensuring seamless communication across different VLANs within the network.