NST21022 - Practical for Network Switching and Routing

Department of Information and Communication Technology

Faculty of Technology

Lab sheet :14

Reg. Number: SEU/IS/20/ICT/084

Academic Year :2020/2021

Practical No :14

Title: Configure DHCPv4

Aim:

● Getting familiar with DHCPv4

Task:

● Configure a Router as a DHCPv4 server

● Configure DHCPv4 relay

● Verify DHCPv4 connectivity

Activities

Use “NST21022 Labsheet 14.pka” file

Addressing Table

| Device | Interface | IPv4 Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | G0/0/0 | 192.168.1.1 | 255.255.255. | N/A |
|  | S0/1/0 | 192.168.4.2 | 255.255.255.0 |  |
| R2 | G0/0/0 | 192.168.2.1 | 255.255.255.0 |  |
|  | S0/1/0 | 192.168.4.1 | 255.255.255.0 |  |
|  | S0/1/1 | 192.168.5.1 | 255.255.255.0 |  |
| R3 | G0/0/0 | 192.168.3.1 | 255.255.255.0 |  |
|  | S0/1/1 | 192.168.5.2 | 255.255.255.0 |  |
| DNS-Server | NIC | 192.168.2.10 | 255.255.255.0 | 192.168.2.1 |
| PC-A | NIC | DHCP | DHCP | DHCP |
| PC-B | NIC | DHCP | DHCP | DHCP |
| PC-C | NIC | DHCP | DHCP | DHCP |
| PC-D | NIC | DHCP | DHCP | DHCP |

Exercise 01: Configure a Router as a DHCPv4 server

- Configure IP addresses on each device’s according to addressing table.(Except DNS-Server)

R1>enable

R1#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

R1(config)#

R1(config)#interface g0/0/0

R1(config-if)#ip address 192.168.1.1 255.255.255.0

R1(config-if)#no shutdown

R1(config-if)#exit

R1(config)#

R1(config)#interface s0/1/0

R1(config-if)#ip address 192.168.4.2 255.255.255.0

R1(config-if)#no shutdown

R2>enable

R2#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

R2(config)#

R2(config)#interface s0/1/0

R2(config-if)#ip address 192.168.4.1 255.255.255.0

R2(config-if)#no shutdown

R2(config-if)#exit

R2(config)#

R2(config)#interface s0/1/1

R2(config-if)#ip address 192.168.5.1 255.255.255.0

R2(config-if)#no shutdown

R3>enable

R3#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

R3(config)#

R3(config)#

R3(config)#interface g0/0/0

R3(config-if)#ip address 192.168.3.1 255.255.255.0

R3(config-if)#no shutdown

R3(config-if)#exit

R3(config)#

R3(config)#interface s0/1/1

R3(config-if)#ip address 192.168.5.2 255.255.255.0

R3(config-if)#no shutdown

- Configure R2 to exclude the first 10 addresses from the R1 LAN.

R2(config)# ip dhcp excluded-address 192.168.1.1 192.168.1.10

R2(config-if)#ip dhcp excluded-address 192.168.1.1 192.168.1.10

R2(config)#

R2(config)#ip dhcp excluded-address 192.168.3.1 192.168.3.10

R2(config)#

- Configure R2 to exclude the first 10 addresses from the R3 LAN.

- Create a DHCP pool in R2 for R1 LAN named R1-LAN (Case Sensitive)

R2(config)# ip dhcp pool R1-LAN

R1(config)#ip dhcp pool R1-LAN

R1(dhcp-config)#

R1(dhcp-config)#ip dhcp pool R3-LAN

R1(dhcp-config)#

- Create a DHCP pool in R2 for R3 LAN named R3-LAN

R3>en

R3#config t

Enter configuration commands, one per line. End with CNTL/Z.

R3(config)#ip dhcp pool R1-LAN

R3(dhcp-config)#

- Configure the DHCP pool to include the network address, the default gateway, and the IP address of the DNS server.

R2(dhcp-config)# network 192.168.1.0 255.255.255.0

R2(dhcp-config)# default-router 192.168.1.1

R2(dhcp-config)# dns-server 192.168.2.10

R2(dhcp-config)#network 192.168.1.0 255.255.255.0

R2(dhcp-config)#default-router 192.168.1.1

R2(dhcp-config)#dns-server 192.168.2.10

R2(dhcp-config)#network 192.168.3.0 255.255.255.0

R2(dhcp-config)#default-router 192.168.3.1

R2(dhcp-config)#dns-server 192.168.2.10

- Configure R1 and R3 as a DHCP relay agent

R1(config)# interface g0/0

R1(config-if)# ip helper-address 192.168.4.1

R1(config)#

R1(config)#interface s0/1/0

R1(config-if)#ip address 192.168.4.2 255.255.255.0

R1(config-if)#no shutdown

R1(config-if)#exit

R1(config)#

R1(config)#interface g0/0/0

R1(config-if)#ip helper-address 192.168.4.1

R1(config-if)#no shutdown

R1(config-if)#exit

R1(config)#

R3(config)#

R3(config)#interface g0/0/0

R3(config-if)#ip helper-address 192.168.5.1

R3(config-if)#no shutdown

R3(config-if)#exit

R3(config)#

- Configure hosts to receive their IP addresses from a DHCP server

- Verify host IP addresses and connectivity

Discussion

- In this lab session, we focused on configuring DHCPv4 to dynamically assign IP addresses to devices on a network. First, we set up a router to function as a DHCPv4 server, defining the IP address pool, default gateway, and DNS settings to be provided to clients. Next, we configured DHCPv4 relay on an intermediary router to forward DHCP requests from clients in different subnets to the central DHCP server. Finally, we verified DHCPv4 connectivity by ensuring that the devices on the network received IP addresses automatically from the server, confirming successful configuration and communication between the clients, DHCP server, and relay.