NST21022 - Practical for Network Switching and Routing

Department of Information and Communication Technology

Faculty of Technology

Lab sheet :15

Reg. Number: SEU/IS/20/ICT/084

Academic Year :2020/2021

Practical No :15

Title: Configure DHCPv6

Aim:

● Getting familiar with DHCPv6

Task:

● Configure a Router as a DHCPv6 server

● Configure DHCPv6 relay

● Verify DHCPv6 connectivity

Activities

Use “NST21022 Lab sheet 15.pka” file

Addressing Table

| Device | Interface | IPv6 Address / Prefix |
| --- | --- | --- |
| R1 | G0/0/0 | 2001:DB8:ACAD:2::1/64 |
|  |  | FE80::1 |
|  | G0/0/1 | 2001:DB8:ACAD:1::1/64 |
|  |  | FE80::2 |
| R2 | G0/0/0 | 2001:DB8:ACAD:2::2/64 |
|  |  | FE80::3 |
|  | G0/0/1 | 2001:DB8:ACAD:3::1 |
|  |  | FE80::4 |
| PC-A | NIC | DHCP |
| PC-B | NIC | DHCP |
| DNS Server | NIC | 2001:db8:acad::254 |

Exercise 01: Configure a Router as a DHCPv4 server

- Configure IP addresses on each device’s according to addressing table.(Except DNS-Server)

R1#enable

R1#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

R1(config)#hostname R1

R1(config)#

R2#enable

R2#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

R2(config)#

R2(config)#hostname R2

R2(config)#

- Enable IPv6 routing in both routers

R1(config)# ipv6 unicast-routing

R1(config)#ipv6 unicast-routing

R1(config)#

R2(config)#ipv6 unicast-routing

R2(config)#

- Create a stateless DHCP (SLAAC) pool in R1 for R1 LAN named R1-STATELESS (Case Sensitive)

R1(config)# ipv6 dhcp pool R1-STATELESS

R1(config-dhcp)# dns-server 2001:db8:acad::254

R1(config-dhcp)# domain-name stateless.com

R1#configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

R1(config)#

R1(config)#interface g0/0/1

R1(config-if)#ipv6 nd other-config-flag

R1(config-if)#ipv6 dhcp server R1-STATELESS

R1(config-if)#exit

R1(config)#

R1(config)#interface g0/0/0

R1(config-if)#ipv6 dhcp server R2-STATEFUL

R1(config-if)#no shutdown

R1(config-if)#exit

- Configure the G0/0/1 interface on R1 to provide the other config flag to the R1 LAN and assign the DHCP pool.

R1(config)# interface g0/0/1

R1(config-if)# ipv6 nd other-config-flag

R1(config-if)# ipv6 dhcp server R1-STATELESS

R1(config)#ipv6 dhcp pool R1-STATELESS

R1(config-dhcpv6)#dns-server 2001:db8:acad::254

R1(config-dhcpv6)#domain-name stateless.com

R1(config-dhcpv6)#exit

R1(config)#

R1(config)#

- Enable IPv6 DHCP on PC-A

- Examine the output of ipconfig /all and notice the changes.

- Create a stateful DHCPv6 pool in R1 for R2 LAN named R2-STATEFUL (Case Sensitive)

R1(config)# ipv6 dhcp pool R2-STATEFUL

R1(config-dhcp)# address prefix 2001:db8:acad:3::/64

R1(config-dhcp)# dns-server 2001:db8:acad::254

R1(config-dhcp)# domain-name stateful.com

R1(config)#ipv6 dhcp pool R1-STATELESS

R1(config-dhcpv6)#dns-server 2001:db8:acad::254

R1(config-dhcpv6)#domain-name stateless.com

R1(config-dhcpv6)#exit

R1(config)#

R1(config)#

- Assign the DHCPv6 pool you just created to interface g0/0/0 on R1.

R1(config)# interface g0/0/0

R1(config-if)# ipv6 dhcp server R2-STATEFUL

R1(config)#ipv6 dhcp pool R2-STATEFUL

R1(config-dhcpv6)#address prefix 2001:db8:acad:3::/64

R1(config-dhcpv6)#dns-server 2001:db8:acad::254

R1(config-dhcpv6)#domain-name stateful.com

R1(config-dhcpv6)#exit

- Configure R2 as an IPv6 dhcp relay agent

R2(config)# interface g0/0/1

R2(config-if)# ipv6 nd managed-config-flag

R2(config-if)# ipv6 dhcp relay destination 2001:db8:acad:2::1 g0/0/0

Note: relay destination command not work for Packet Tracer but work on real router.

R1(config)#interface g0/0/0

R1(config-if)#ipv6 address 2001:db8:acad:2::1/64

R1(config-if)#ipv6 address fe80::1 link-local

R1(config-if)#no shutdown

R1(config-if)#exit

R1(config)#

R1(config)#

R1(config)#interface g0/0/1

R1(config-if)#ipv6 address 2001:db8:acad:1::1/64

R1(config-if)#ipv6 address fe80::2 link-local

R1(config-if)#no shutdown

R2(config)#interface g0/0/1

R2(config-if)#ipv6 nd managed-config-flag

R2(config-if)#ipv6 dhcp relay destination 2001:db8:acad:2::1 g0/0/0

^

% Invalid input detected at '^' marker.

R2(config-if)#no shutdown

R2(config-if)#exit

%LINK-5-CHANGED: Interface GigabitEthernet0/0/0, changed state to up

%LINEP

- Verify host IPv6 addresses and connectivity.

Discussion

- In this lab session, we worked on setting up DHCPv6 to provide dynamic IPv6 address assignment across the network. First, we configured a router to act as a DHCPv6 server, specifying the IPv6 address pool and other network settings to be assigned to clients. Then, we enabled DHCPv6 relay on another router to forward client requests from different subnets to the DHCPv6 server. Finally, we verified the connectivity by checking if the clients received their IPv6 addresses automatically from the server, ensuring that the DHCPv6 server and relay configurations were correctly set up and functional.