# Lab Sheet – 16

Title: Configure EtherChannel, HSRP and Switch Security

Aim:

- Configure EtherChannel

- Configure HSRP (Hot Standby Router Protocol)

- Configure Switch Security

Task:

- Configure an EtherChannel with Cisco PAgP

- Configure an 802.3ad LACP EtherChannel

- Configure a Redundant EtherChannel Link

- Configure an HSRP active router

- Configure an HSRP standby router

- Secure Trunk

- Secure Unused Switchports

- Implement Port Security

- Enable DHCP Snooping

- Configure Rapid PVST PortFast and BPDU Guard

Activities

Use “NST21022 Labsheet 16.pka” file

Addressing Table

| Device | Interface | IP Address | Default Gateway |
| --- | --- | --- | --- |
| R1 | G0/0/0 | 192.168.1.1 | N/A |
|  | S0/1/0 | 192.168.3.1 |  |
|  | S0/1/1 | 192.168.2.1 |  |
| R2 | G0/0/0.10 | 192.168.5.1 | N/A |
|  | G0/0/0.20 | 192.168.6.1 |  |
|  | S0/1/0 | 192.168.3.2 |  |
|  | S0/1/1 | 192.168.4.1 |  |
| R3 | G0/0/0.30 | 192.168.7.1 | N/A |
|  | G0/0/0.40 | 192.168.8.1 |  |
|  | S0/1/0 | 192.168.2.2 |  |
|  | S0/1/1 | 192.168.4.2 |  |
| HSRP Virtual Gateway | Virtual | 192.168.6.254 |  |
| Web Server | NIC | 192.168.1.10 | 192.168.1.1 |
| PC-1 | NIC | 192.168.5.10 | 192.168.5.1 |
| PC-2 | NIC | 192.168.8.10 | 192.168.8.1 |
| PC-3 | NIC | 192.168.6.10 | 192.168.6.1 |
| PC-4 | NIC | 192.168.7.10 | 192.168.7.1 |

Exercise 01: Configure EtherChannel

| Channel Group | Ports | Protocol |
| --- | --- | --- |
| 1 | S1 F0/11, F0/12 S3 F0/11, F0/12 | PAgP |
| 2 | S2 F0/11, F0/12 S4 F0/11, F0/12 | LACP |
| 3 | S3 F0/21, F0/22 S4 F0/21, F0/22 | Negotiated LACP |

- Configure an EtherChannel with Cisco PAgP on S1 and S3

S1(config)# interface range f0/11– 12

S1(config-if-range)# shutdown

S1(config-if-range)# channel-group 1 mode desirable

S1(config-if-range)# no shutdown

S3(config)# interface range f0/11-12

S3(config-if-range)# shutdown

S3(config-if-range)# channel-group 1 mode desirable

S3(config-if-range)# no shutdown

- Configure the logical interface to become a trunk by first entering the interface port-channel number command and then the switchport mode trunk command

S1(config)# interface port-channel 1

S1(config-if)# switchport mode trunk

S3(config)# interface port-channel 1

S3(config-if)# switchport mode trunk

- Configure an 802.3ad LACP EtherChannel on S2 and S4

S2(config)# interface range f0/11 - 12

S2(config-if-range)# shutdown

S2 (config-if-range)# channel-group 2 mode active

S2(config-if-range)# no shutdown

S2(config-if-range)# interface port-channel 2

S2(config-if)# switchport mode trunk

- Configure a Redundant EtherChannel Link on S3

S3(config)# interface range f0/21 - 22

S3(config-if-range)# shutdown

S3(config-if-range)# channel-group 3 mode passive

S3(config-if-range)# no shutdown

S3(config-if-range)# interface port-channel 3

S3(config-if)# switchport mode trunk

- On S4, add ports F0/21 and F0/22 to Port Channel 3 with the channel-group 3 mode active command.

Exercise 02: Configure HSRP (Hot Standby Router Protocol)

- Trace the path to the Web Server from PC-1

- Trace the path to the Web Server from PC-3

- Configure HSRP on R3

R3(config)# interface g0/0/0

- Specify the HSRP protocol version number. The most recent version is version Note: Standby version 1 only supports IPv4 addressing

R3(config-if)# standby version 2

- Configure the IP address of the virtual default gateway

R3(config-if)# standby 1 ip 192.168.6.254

- Designate the active router for the HSRP group

R3(config-if)# standby 1 priority 150

- R3 will operate as the active router and traffic from the two LANs will use it as the default gateway

R1(config-if)# standby 1 preempt

- Repeat step 4, 5 on R2 G0/0/0 interface to make standby router

- Verify HSRP configuration using tracert command

Exercise 03: Configure Switch Security

| Switch | VLAN Number | VLAN Name | Port Member | Network |
| --- | --- | --- | --- | --- |
| S1 | 100 | Native | -- | -- |
| S3 | 10 | Sales | F0/1 | 192.168.5.0/24 |
|  | 20 | Production | F0/2 | 192.168.6.0/24 |
|  | 999 | BlackHole | Unused | -- |
| S4 | 30 | Accounts | F0/2 | 192.168.8.0/24 |
|  | 40 | Finance | F0/1 | 192.168.7.0/24 |

- Disable DTP negotiation on S1 G0/1 interface.

S1(config-if)#switchport nonegotiate

- Create VLAN 100 and give it the name Native on S1 and Configure G0/1 trunk port use VLAN 100 as the native VLAN.

S1(config)#vlan 100

S1(config-vlan)#name Native

S1(config)#interface GigabitEthernet0/1

S1(config-if)#switchport trunk native vlan 100

- Secure Unused Switchport

  - Shutdown all unused switch ports on S3.

S3(config)#interface range F0/3-10, F0/13-20, F0/23-24, G0/1-2

S3(config-if-range)#shutdown

S3(config-if-range)#exit

  - On S3, create a VLAN 999 and name it BlackHole. The configured name must match the requirement exactly.

S3(config)#vlan 999

S3(config-vlan)#name BlackHole

S3(config-vlan)#exit

  - Move all unused switch ports to the BlackHole VLAN.

S3(config)#interface range F0/3-10, F0/13-20, F0/23-24, G0/1-2

S3(config-if-range)#switchport access vlan 999

- Implement Port Security

  - Activate port security on all the active access ports on switch S3

S3(config)#interface range FastEthernet0/1-2

S3(config-if-range)#switchport mode access

S3(config-if-range)#switchport port-security

  - Configure the active ports to allow a maximum of 4 MAC addresses to be learned on the ports.

S3(config)#interface range FastEthernet0/1-2

S3(config-if-range)#switchport port-security maximum 4

  - For ports F0/1 on S3, statically configure the MAC address of the PC using port security.

S3(config)#interface FastEthernet0/1

S3(config-if)#switchport port-security mac-address <<PC-Add>>

  - Configure each active access port so that it will automatically add the MAC addresses learned on the port to the running configuration.

S3(config)#interface range FastEthernet0/1-2

S3(config-if-range)#switchport port-security mac-address sticky

  - Configure the port security violation mode to drop packets from MAC addresses that exceed the maximum, generate a Syslog entry, but not disable the ports.

S3(config)#interface range FastEthernet0/1-2

S3(config-if-range)#switchport port-security violation restrict

- Configure DHCP Snooping

  - Configure the trunk ports on S3 as trusted ports.

S3(config)#interface range FastEthernet0/11-12

S3(config-if-range)#ip dhcp snooping trust

  - Limit the untrusted ports on S3 to five DHCP packets per second.

S3(config)#interface range FastEthernet0/1-2

S3(config-if-range)#ip dhcp snooping limit rate 5

  - On S4, enable DHCP snooping globally and for VLANs 30 and 40.

S4(config)#ip dhcp snooping

S4(config)#ip dhcp snooping vlan 30,40

- Configure PortFast, and BPDU Guard

  - Enable PortFast on all the access ports that are in use on S3.

S3(config)#interface range FastEthernet0/1-2

S3(config-if-range)#spanning-tree portfast

  - Enable BPDU Guard on all the access ports that are in use on S3.

S3(config)#interface range FastEthernet0/1-2

S3(config-if-range)#spanning-tree bpduguard enable

  - Configure S4 so that all access ports will use PortFast by default.

S4(config)#spanning-tree portfast default